10 GOSUB 90

20 PRINT" START"

30 GOSUB 100

40 *SAVE SRLRUN.COM E900 EA34

70 STOP

90 BEGIN=&E900

100 CODE=&E900

110 P%=CODE

120 [OPT 1

130 .BEGIN LD BC,&0132; load BC the number of bytes to move.

140 LD DE,&E912; DE the address to move to.

150 LD HL,&0112; HL the address to move from

160 LDIR

170 JP LAD_ADDS; jump and run srlrun.com from its new location &E916.

180 NOP

190 NOP

200 NOP

210 RET

220 .RN DEFW &0100; store address to download starting at.

230 .SAFE DEFW &E8FE; fail safe to stop downloading over running SRLRUN.COM.

240 .LAD_ADDS LD HL,(SAFE);

250 LD DE,(RN) ;

260 CALL CHK_IN ; a routine to clear serial port buffer.

270 .CHK_START CALL RTS_ON ; handshaking allow PC to download.

280 CALL RECEIVE ; a routine to wait for next byte from serial input.

290 IN A,(&10) ; put serial input into A reg.

300 LD DE,(RN) ; load DE with address to load at.

310 CALL RTS_OFF ; set RTS on to hold flow of serial input from PC.

320 CP 126 ; compare A to  ~" ascii 126

330 CALL Z,LAD_START ; routine to change DN store according to next 4   hex ascii bytes from serial input.

340 CP 92 ; compare A reg with  /

350 LD DE,(RN) ; load DE new start address if changed.

360 CALL Z,LAD_PROG ; routine that converts and loads hex list into Einstein.

370 JR CHK_START ; loop back

380 RET

390 .CHK_IN PUSH AF ; routine to clear bytes not wanted for loading.

400 CALL RTS_ON

410 .CHKO IN A,(&10);clear buffer

420 IN A,(&11) ; read 8521A USART register.

430 BIT 1,A ; see if bit 1 = 0.

440 JR NZ, CHKO ; if not jump back and clear next byte.

450 POP AF

460 RET

470 .LAD_START PUSH AF ;routine to change address where code will be loaded.

480 CALL  GT_BYTE ; download next two ascii hex bytes convert to real byte

490 LD D,A ; real byte returned in A reg. then loaded into D reg.

500 CALL H_PRINT ; routine convert real byte back to ascii and eco to PC terminal.

510 CALL GT_BYTE ; get next real byte.

520 LD E,A ; put second byte into E reg.

530 CALL H_PRINT ; echo second byte to PC.

540 LD (RN),DE ; store DE for new address to down load to.

550 POP AF

560 RET

570 .LAD_PROG ADD A,00 ; routine to load code into Einstein.

580 DEC A

590 DEC A

600 PUSH HL

610 PUSH DE

620 SBC HL,DE ; check to see if loading over runs.

630 JR Z NO_START ; if so jump to abort

640 POP DE

650 POP HL

660 CALL GT_BYTE; GET TWO ASCI HEX BYTES FOR REAL BYTE

670   LD (DE),A ; returned byte in A reg. loaded at address given by DE

680   CALL ECO ; echo loaded byte back to PC terminal.

690   INC DE ; get next address to load next byte.

700   JR LAD_PROG ; loop back

710   RET

720   .NO_START RET ; fail safe finish.

730   .GT_BYTE PUSH HL ; the routine to download two hex ascii bytes for real byte.

740   PUSH DE

750   CALL RTS_ON ; set handshaking on.

760   .BACK CALL RECEIVE ; routine to check if new byte has been received in 8251A buffer.

770   IN A,(&10) ; returned new byte in A reg.

780   CP 71 ; compare A to  G

790   JR NC,BACK ; if A greater than 71-ascii G or higher jump back get next byte.

800   CP 48 ; compare A to  0

810   JR C,BACK ; if A smaller than 48 jump back and get next byte.

820   CALL RTS_OFF ; hardware handshaking to hold down load from PC.

830   CP 61 ; compare A reg.(next byte) to  =

840   JR Z,INEND ; if so program down load complete jump to end.

850   CP &40 ; compare A to  @  ascii character before   A

860   JR C,WR1 ; jump if A value less than &40-for digits 1 to 9

870   AND &DF

880   SUB 07 ; standardize character code.

890   .WR1 ADD A,A ; shift A one hex digit left

900   ADD A,A ; so &30 becomes 00

910   ADD A,A

920   ADD A,A

930   PUSH HL ; store HL

940   LD L,00

950   LD H,00

960   LD H,A ; H=first hex digit * 16

970   CALL RTS_ON ; hand shaking allows serial input

980   .BACK2 CALL RECEIVE ; checks for new byte received.

990   IN A,(&10) ; serial input returned in A reg.

1000   CP 71 ; compare A to  G .

1010   JR NC,BACK2 ; if G or Higher jump back and get next byte.

1020   CP 48 ; compare A to  0

1030   JR C,BACK2 ; if not  0  and ascii characters less than 48.

1040   CALL RTS_OFF ; hand shaking hold serial input.

1050   CP 61 ; compare A reg. to  = .

1060   JR Z,INEND ; if so program download complete jump to end.

1070   CP &40 ;compare A to  @  ascii character before   A

1080   JR C,WR2

1090   AND &DF

1100   SUB 07

1110   .WR2 AND &0F ; consider second hex digit only.

1120   OR H ; combine with first hex digit-A reg now holds value of two ascii hex bytes.

1130   POP HL

1140   POP DE

1150   POP HL

1160   RET

1170   .ECO PUSH DE ;routine to echo byte loaded back to PC terminal.

1180   PUSH HL ; store HL

1190   LD H,D:LD L,E ; put address held in DE into HL.

1200   PUSH AF

1210   LD A,(HL) ; load last transferred byte into A reg.

1220   CALL H_PRINT ; routine to serial output A reg. value.

1230   POP AF

1240   POP HL ; restore HL value.

1250   POP DE

1260   RET

1270   .RECEIVE PUSH AF ; routine to check for new byte in 8251 USART.

1280   .CHK IN A,(&11)

1290   BIT 1,A ; bit 1=1 if fresh byte received.

1300   JR Z, CHK ; if bit 1=0 go back for new byte

1310   POP AF

1320   RET

1330   .RTS_ON PUSH AF ; hardware handshaking routine.

1340   LD A,&27 ; bits 0,1,4 and 5 set. bit 5 is request to send enabled.

1350   OUT (&11),A ; out A to 8521A reg.

1360   POP AF

1370   RET

1380   .RTS_OFF PUSH AF ; hardware handshaking routine to hold serial out put from PC.

1390   LD A,&07 ; bits 0,1,2 set and bit 5 reset to 0

1400   OUT (&11),A ;out A to 8521A reg.

1410   POP AF

1420   RET

1430   .END2 POP HL

1440   .INEND CALL CHK_IN

1450   POP HL ; restore as jumped out of loop.

1460   POP DE; restore as jumped out of loop.

1470   POP AF; restore as jumped out of loop.

1480   LD A,64; load A  ascii  @

1490   CALL SEND; send to pc terminal to denote download complete.

1500   LD HL,&0100 ;

1510   LD (RN),HL ; reset load start address

1520   .LEP JP &0100 ; jump to run com prog.-change to hex 0000 to reboot and not run.

1530   RET

1540   .H_PRINT PUSH AF ; routine to turn real byte into two hex ascii bytes.

1550   AND &F0 ; isolates first digit.

1560   RRA ; move this

1570   RRA ; digit to

1580   RRA ; its proper position

1590   RRA ; in A reg.

1600   ADD A,&30 ; change to ascii character.

1610   CP &3A ; is digit between A and FOR

1620     JR C,HP_H ;

1630     ADD A,07 ; change to correct symbol if so.

1640     .HP_H CALL SEND ; serial output byte.

1650     POP AF ; retrieve original value AND

1660     AND &0F ; isolate second digit.

1670     ADD A,&30 ; change to ascii value for character.

1680     CP &3A ;

1690     JR C,HP_L

1700     ADD A,07 ; change to correct hex symbol.

1710     .HP_L CALL SEND ; serial output AND

1720     RET

1730     RET

1740     .SEND PUSH AF ; store A value

1750     .REP IN A,(&11)) ; read 8251A usart reg.

1760     BIT 0,A ; bit 0=1 if new byte ready to send.

1770     JR Z  REP ; jump back if byte not ready.

1780     POP AF ; restore A

1790     OUT (&10),A ; serial output A

1800     RET

1810     ]

1820     RETURN

>