21 July 1996

The boot record is NOT executable.  Any attempt to execute it will produce

varied effects ranging from a HALT to only a single load or jump.  The

arguments of the instructions are simply intended to make addresses available

to the ROM Boot loader.  The layout is:

ORG	8000H

DB	76H		; HALT instruction

LD	HL,CCP-80H	; Base load point assuming 128 bytes for

;  this Boot Record size.

RET			;   (safety)

JP	BIOS		; This address is for the CBOOT entry in the

;  relocated System image.  Execution should

;   commence at the argument of this jump.

Accessing the relevant parameters from the ROM, then, normally consists of

getting the Load Address from 8002H, and the BIOS CBOOT Entry point from

8006H.  It is recommended that the jump itself not be executed, but use a

vectored approach such as the sequence:

LD	HL,(8006H)

JP	(HL)

The remainder of this record is available for transferring parameters to the

Bios as part of the CBOOT setup.  Parameters may be addressed by absolute

values such as 8020H, etc.

Since the two location addresses are variable depending on the system size

generated, a checksum of the boot record is not feasible without generating

an entirely new, unique utility.  That defeats the intent of this effort which

is to retain a common suite of utilities across multiple platforms.  Validation

of the Boot record instead can check the absolute bytes at any or all of the

following locations:

8000 - 76H

8001 - 21H

8004 - 0C9H

8005 - 0C3H

Another possibility which may be implemented is to embed an ASCII Text string

in the available space.  This may be a visual thing, but there comes a limit

to complexity vs reliability.

Hal

#############################################